# Wireshark Network Traffic Flow Analysis: Comprehensive Lab Report

Student Name: Ibrahim Al-ameen (CYBERHIM)

Course/Assignment: Network Traffic Flow Analysis Practice

Environment: Kali Linux (Virtual Machine), Wireshark, Command Line Tools (curl, ping)

## Executive Summary

This document provides a thorough, step-by-step breakdown of network traffic capture and analysis across various layers of the OSI model. By examining live packets using Wireshark and network command-line utilities, this lab maps out the foundational transition of web traffic from unencrypted protocols to secure, encrypted transport layers.

## Phase 1: Initial Setup, Environment Verification, & Capture Initialization (Steps 1 – 5)

- Step 1: Interface Selection and Promiscuous Mode Initialization
Action: Launched Wireshark within the Kali Linux virtual machine and initialized packet capture on the primary active network interface (eth0 / wlan0).
Observation: Confirmed that promiscuous mode was enabled to capture all packets traversing the local network segment, not just those destined for the host machine.
Associated Artifact: Screenshots/ASSIGNMENT1.png

- Step 2: Baseline Traffic Observation
Action: Allowed the capture to run idle for a brief period to log background multicast and broadcast traffic.
Observation: Identified standard ARP (Address Resolution Protocol) requests and local router discovery packets populating the packet list pane.
Associated Artifact: Screenshots/ASSIGNMENT2.png

- Step 3: Generating Targeted ICMP Traffic (Ping)
Action: Executed a ping command from the terminal targeting a public IP address (ping -c 4 8.8.8.8) to generate predictable network events.
Observation: Isolated the ICMP Echo Request (Type 8) and Echo Reply (Type 0) frames inside Wireshark.
Associated Artifact: Screenshots/Assignment 3.png

- Step 4: Applying Packet Display Filters
Action: Typed icmp into the Wireshark display filter bar and applied it.
Observation: Stripped away all background noise, leaving only the ICMP round-trip transactions visible for analysis.
Associated Artifact: Screenshots/Assignment4.png

- Step 5: Examining ICMP Packet Details
Action: Clicked into an individual Echo Request packet and expanded the Internet Control Message Protocol layer in the packet details pane.
Observation: Inspected the payload data, sequence numbers, and time-to-live (TTL) fields set by the operating system.
Associated Artifact: Screenshots/Assignment 5.png

## Phase 2: DNS & Domain Resolution Analysis (Steps 6 – 10)

- Step 6: Triggering a DNS Query
Action: Cleared previous filters, started a fresh capture stream, and executed nslookup example.com in the terminal.
Observation: Captured the immediate generation of outbound User Datagram Protocol (UDP) packets directed toward port 53 (DNS).
Associated Artifact: Screenshots/Assignment6.png

- Step 7: Filtering for DNS Protocol
Action: Applied the dns display filter in Wireshark to isolate name resolution traffic.
Observation: Separated the transaction into distinct query and response frames.
Associated Artifact: Screenshots/Assignment7.png

- Step 8: Analyzing the DNS Standard Query (A Record)
Action: Expanded the Domain Name System (Query) layer in packet details.
Observation: Verified that the query flags indicated a standard query (Standard query) asking for the IPv4 Address (A record) of the target domain.
Associated Artifact: Screenshots/Assignment8.png

- Step 9: Analyzing the DNS Standard Response
Action: Inspected the corresponding DNS response packet returned from the local/public resolver.
Observation: Located the Answers section within the packet structure, confirming the successful resolution of the domain name to its corresponding public IP address.
Associated Artifact: Screenshots/Assignment9.png

- Step 10: UDP Transport Layer Inspection for DNS
Action: Inspected the Transport Layer (User Datagram Protocol) headers wrapping the DNS payload.
Observation: Noted source and destination ports, verifying that DNS operates statelessly over UDP port 53 for fast query turnaround.
Associated Artifact: Screenshots/Assignment10.png

## Phase 3: TCP 3-Way Handshake & Connection Streams (Steps 11 – 15)

- Step 11: Initiating an HTTP Connection
Action: Targeted a test web server using curl to initiate a fresh connection session.
Observation: Captured the rapid initialization sequence of packets preceding application data exchange.
Associated Artifact: Screenshots/Assignment11.png

- Step 12: Isolating TCP Traffic
Action: Applied the tcp display filter to examine transport-level segments.
Observation: View populated with connection establishment flags and sequence tracking values.
Associated Artifact: Screenshots/Assignment12.png

- Step 13: Capturing the TCP SYN Packet
Action: Selected the initial packet sent from the client.
Observation: Verified the presence of the [SYN] flag and an initial Sequence Number (ISN) set to zero (relative).
Associated Artifact: Screenshots/Assignment13.png

- Step 14: Capturing the TCP SYN-ACK Response
Action: Selected the server's reply packet.
Observation: Identified both the [SYN] and [ACK] flags set simultaneously, along with an Acknowledgement Number incremented to validate the client's initial sequence.
Associated Artifact: Screenshots/Assignment14.png

- Step 15: Completing the Handshake (ACK) & Following the Stream
Action: Located the final client [ACK] packet, right-clicked it, and selected Follow > TCP Stream.
Observation: Opened the stream dialog to view the raw exchange of data frames occurring over this established circuit.
Associated Artifact: Screenshots/Assignment15.png

## Phase 4: Unencrypted HTTP Traffic & Payload Extraction (Steps 16 – 17)

- Step 16: Forcing Plaintext HTTP Request
Action: Used curl http://... specifically on an unencrypted endpoint to bypass automatic HTTPS redirection.
Observation: Captured an unencrypted GET request packet directed to TCP port 80.
Associated Artifact: Screenshots/Assignment16.png

- Step 17: Extracting HTTP Payload Content
Action: Followed the TCP stream of the HTTP 200 OK response packet.
Observation: Inspected the raw plaintext body containing structural HTML tags, server headers, and content types, confirming that data passed over port 80 is fully readable to anyone monitoring the medium.
Associated Artifact: Screenshots/Assignment17.png

## Phase 5: Secure TLS Handshake & Encrypted Data Analysis (Steps 18 – 21)

- Step 18: Initiating a Secure HTTPS Connection
Observation: Logged the transition of traffic to TCP port 443 followed by specialized Transport Layer Security negotiation frames.
Associated Artifact: Screenshots/Assignment18.png

- Step 19: Analyzing the TLS Client Hello
Action: Expanded the Transport Layer Security protocol layer on the initial outgoing secure handshake packet.
Observation: Inspected the supported TLS version list, random client bytes, and the extensive array of proposed Cipher Suites available for session encryption.
Associated Artifact: Screenshots/Assignment19.png

- Step 20: Examining the Server Certificate & Handshake Conclusion
Action: Inspected the server's reply frame containing the Server Hello, Certificate chain, and Key Exchange parameters.
Observation: Verified the digital certificate details (Issuer, Subject CN, and validity period) used by the server to prove its identity to the client.
Associated Artifact: Screenshots/Assigement20.png / Screenshots/Assignment20.png

- Step 21: Analyzing Encrypted Application Data
Action: Navigated past the completion of the cryptographic handshake to observe post-handshake application packets.
Observation: Confirmed that payload data was classified strictly as Encrypted Application Data—appearing as unreadable ciphertext rather than plaintext bytes, proving the privacy benefits of modern cryptographic tunneling.
Associated Artifact: Screenshots/Assignment21.png

## Conclusion & Security Comparison

The side-by-side analysis of HTTP and TLS streams underlines critical cybersecurity realities:

- HTTP (Plaintext): Leaves application layer payloads exposed to trivial inspection, extraction, and tampering by malicious internal or external actors on the network path.

- TLS (Encrypted): Protects data confidentiality and integrity via robust key exchanges and cipher suites, ensuring that even if packets are intercepted, the contents remain completely secure.